All about lubricants

By Charlie

Revised 2/12/12

This article is about the various lubricants and cleaners, both home brew and commercially available, so that you can determine for yourself what you need and where to get them.

Differences and uses for Maccari Tars

Jim Maccari’s lubes along with springs and seals are available at:

http://www.airguns.citymax.com/page/page/251327.htm 

Heavy tar: For use on spring OD. Light cover and string across the coils of spring. Lubing the entire length is not necessary.  Lube the section of the spring that is in the piston when compressed, or about ¾ of the spring length.

Clear Tar:  triggers, pivot points etc. Best product for the OD of spring guides. Great for use on pistons and compression tubes on side levers or under levers like the TX, PS, and RWS etc. Being clear it does not create a mess. Remove excess. The best lube you can use on guide ID's- use very, very little.

Gene Sundays Mystery Oil ….

Take one tablespoon of Moly grease and mix it thoroughly with two ounces of 30wt non-detergent motor oil. Shake well. Serve as needed. Use sparingly on any metal to metal surfaces. Remember, a little of Moly goes a long way. Good for trigger assemblies and trigger pivots, also any externally lubed areas such as pivot points.

Charlie’s Leather Seal Sauce…… (Chamber Lube)……

Mix 1 part NeatsFoot oil to 3 parts 100% Silicone Shock oil.  .  Example: 1 ounce of NeatsFoot oil and 3 ounces of Silicone Oil. Also, ¼ ounce of Teflon can be added if you have it. That will be enough to last for years if used just as a chamber lube. Used for guns having a leather seals.

Silicone Grease

100% Dielectric Silicone Grease is available at almost any auto parts store. If they don’t know what you’re talking about, tell them it’s the stuff used on spark plug boots. Use it on seals, Co2 tubes and valves..many assembly applications. Not to be used as a piston or chamber lube.

100% Silicone Oil

Purchase from any Hobby Center that has Radio Controlled race cars. It comes in many weights just like motor oil but in small quantities. I use 30wt shock oil only. Used in many applications and as a mix for some of the home brews such as Gene Sundays Mystery Oil in place of 30wt non-detergent oil but expensive to use this way.

Moly Paste

Used as a lube on the piston and seal edge, spring guides, pivots, spring ends, cocking slots and many other applications. It is also used in home brews. There are not a lot of known sources for a good Moly lube in small quantities with a minimum 60% ratio (preferably 65%) or higher. Don’t use the cheap low grade stuff from Wally World or parts stores as the moly content is quite low and a waste of money.

Maccari of course has it and Honda does have one with 60% content, although they may have to order it. Keep in mind though is not a “good” replacement for the Maccari molly.

Honda part number…..Moly 60 Paste…….Part Number 08734-0001

Super-Lube

SuperLube w/PTFE is a good lubricant. Great for piston into tube assembly and it has a silicone base with PTFE to help fill imperfections in compression chambers in synthetic sealed guns and tubes, especially in the Chinese made guns. Must be used sparingly as it will cause detonation. Wipe out all excess before installing piston. Carried by many parts stores and I think Wal-Mart.

NeatsFoot Oil

Used as a conditioner for leather seal when mixed with Silicone oil and also as a mix for chamber lubes. NeatsFoot can be hard to find but it is good. If another seal conditioner is used, be sure that it is for real leather and not for synthetics.

RemOil

RemOil is used on the stock and exterior metal as a cleaner/preservative and can be used in the barrel when stored for long periods of time, but in springer’s, needs to be cleaned out with patches before using the gun. It can be found in most sporting goods stores.

GooGone

Used for cleaning the barrel internally. It will break down most chemicals and deposits and at the same time, will not damage the breech seal and won’t harm most finishes but be careful around some plastics.

GooGone can be found just about anywhere.

Loc-Tite

Not a lube but used to secure the stock screws and pivot bolts in the gun making it more solid. It’s available at most hardware stores and Wal-Mart or other department store. Be sure to get and use only the “Blue” Loc-Tite.

Well my friends, that just about covers most of the basic lubes used. Be sure to keep in mind that usually when lubing an airgun, less is better. Do not over lubricate.

Thank You....

Charlie

E-mail:    CDT22@Frontier.com

Web Site:    http://charliedatuna.com/