How to care for your B-4 and KL-B3

By Charlie

Your gun has been lubed both internally and externally during the tuneup process. Internally it should never have to be lubed again unless it is tuned again in the future. For all intents and purposes, it is self lubing and should last for years. Use Rem-Oil on the wood and exterior metal.

Externally it should be lubed about every three thousand rounds or so, or at least once a year. It is a simple procedure. If your gun is put away for long periods of time, it should be fired every three months or so with about 5 shots or so it may lube itself.

The best lube to use is a mixture of moly and 30wt non-detergent oil. Mix it 1 teaspoon of moly to three ounces of oil. Lube very sparingly. I use a tooth pick dipped it the mixture and put only a drop or so. If moly is unavailable, use 30wt non-detergent oil.

First remove the stock by removing the two Phillips screws located in the forestock and the rear screw in the trigger housing. The assembly lifts out of the stock although sometimes it is a tight fit at the trigger end. Use a good Phillips screwdriver as these screws are installed with LocTite during final assembly to keep them from loosening.

Lube all of the pivot points where you see the rivets, the long thin metal piece that goes from the cocking arm to the trigger, three or four drops spread out into the trigger mechanism from the trigger (bottom) side and the little spring, both sides of cocking pivot points and into the slot that you can look into and see the spring and at the cocking foot and a drop or two on the cocking foot itself.

Cleaning the bore

The best thing for cleaning the bore is GooGone. You can find it at just about any hardware store, Wal-Marts and K-marts I think, and places like Home Depot or Lowes.

The best way to do it is to use is a piece of trimmer line, the plastic or nylon line used on grass trimmers, cut to the length that is best for you, heat it on one end to create a small ball and on the other end cut it to a point. Then using .22 bore cleaning patches, push the sharp end of the line through the patch(s), then put some GooGone on the patches and pull them through the bore. Do this several times first with the ones with the GooGone and then dry patches until they come through clean. By the way, the barrel doesn't have to be cleaned very often, maybe every three or four tins of pellets. The lead deposits act as a lube for the pellets.

Note    Note    Note

Initially, the velocity may drop a little but as the seal breaks in and mates to the compression chamber it will increase, usually somewhere between 15 and 25 fps. This is normal. Also, the cocking will get a little easier as will the trigger pull.

Thank you for letting me be of service to you.

Charlie