You can get..
June 19 2003 at 9:44 PM

a standard Chinese replacement syn piston seal from Compasseco, also ask for a screw and spread washer. the 36-2 takes the same seal as the QB-88,B4-1, B-4-2 etc, it just has a longer Spring and air chamber to make more power. When you install seal, as you tighten down screw seal expands and gets tighter, just get it tight enough to have a moderate drag, it will wear in in time. Don’t forget to LocTite threads, you don’t want that bugger falling off! Also go ahead and order the Maccari TF-99 mainspring(same gun as a 36-2) for around $10 plus shipping. Don’t forget to tar the spring*moly ends. My gun shoots Hobby pellets at around 925fps,forget about the 1100fps hype.LOL,DD