Pellet Sizing

Author unknown

Presented by Charlie

One night while shooting some of the cheap, nasty Industry .177 pellets I noticed shots were scattering all around the bullseye of the target. The gun I was using at the time has always shot very accurately in previous sessions and the only difference was the brand of pellet being used. So I dug out the venire (micrometer) and measured the skirts on several pellets! Whoa! They ranged all over the place and some were actually oval.

So I grabbed a piece of steel and drilled a .182" hole in it, hit it with a countersink to give it a bit of a smooth chamfer/lead and began pressing the pellets through it. After 10 or so pellets I gave them a try. Voila! The shot groups became much tighter and very acceptable. Even the gun sounded better...no longer wheezing on some pellets and sneezing on others.

After that I decided that a swage is pretty much a necessity for true accuracy. The folks at the gun club who shoot in competitions use a swage religiously (and a scale too) so this gave some credibility to their actions.

So I took a piece of 2" x 2" x 1/4" wall square tubing about 4" long and drilled a series of holes spaced evenly along one face: .177", .180", .182" and .185" in diameters. Again I used a countersink to chamfer each hole as a lead in. As a tool to press the pellets through I took a 1/8" hardened pin and soldered it into a 3/16" roll pin. Anything with a blunt end will work though...just so long as it fits inside the pellet skirt.

The reason for the different sized holes is that some guns behave differently than others with a given pellet skirt size. I wanted some variation available so I could determine which size shot best with which gun. I don't have any "results" as yet other than it appears that the .185" swage hole seems to work very well with my B-4-2 and KL-3B while the .180" hole seems to work best with both my Daisy 840's.

The swage block/tool worked so well with .177 caliber guns that I flipped the block and drilled holes for the .22 caliber guns: .221", .228" and .234" in diameters. I have no tangible results from those diameters as yet other than I did shoot some .228" in the B-4-2 and the results seemed as good as with the .234" hole in terms of accuracy at 30 feet. I have yet to chrony different skirt diameters though.

Thanks….Charlie