Try this method to improve leather seals

Shot consistencies, due to the overly hard seal not totally conforming to the slight irregularities in the cylinder.
1. Scruff up the outer part of the seal with 180 grit sand paper until all burnish color is removed.
2. Spray seal heavily with brake cleaner to remove all oil and sand paper grit and wipe with cloth and let dry.
3. Soak seal overnight in leather conditioner. 
4. Now grasp seals outer edge between thumb and fore finger and work the leather back and forth while adding drops of leather conditioner until it becomes pliable, NOTE: Make sure to do all the way around the seal, May take 15 minutes for this procedure. 
5. Now repeat step 2 heavily and let completely dry.
6. Now very lightly coat your seal with your factory recommended airgun seal lube or your own personal favorite airgun lube.
Thanks Ray in PA.

| I put syn seal | March 31 2002, 4:28 AM |
| --- | --- |

in my B-4.If you call Compasseco and tell them you need a syn seal for a B4-1 they will send you one and ask for the big washer that will expand seal for moderate drag. It fixed mine with 0 problems. See ya. Sam

Use B3-1 seal in B-4-2 with a washer.