Needle thrust bearings

By CharlieDaTuna

I never use, and no one should use a needle bearing in the piston. There is only one person that I know of that sells that idea and promotes it in a so called tune kit but not one true professional tuner that I know of would or does use them. The kit does work but the parts are designed poorly also and uses what I would call "inferior" lubes but for some, it's an alternative. I never knock them on the forums though except the needle bearing thing.

If they, the bearings, were a good idea, they would be used in guns, especially the expensive ones by the major airgun manufactures. They can actually be counter-productive. The needle bearing thrust is a very good way to sometimes destroy a gun. Needle thrust bearings/washers are designed for a continuous rotating surface with an adjusted constant evenly applied pressure.

Manufactures would never put them in springer airguns because they are not designed for that kind of application and  are not designed to take a constant smashing and hammering that happens inside the springer and have a good chance of breaking up the bearing cage as well as the rollers themselves. Just think of it as laying the bearing on a piece of steel and beating it with 3-400 pounds of impact with a hammer while rotating.